Basler Lens C125-0618-5M-P#

Basler Premium C-mount lens with a fixed focal length of 6 mm, aperture range from F1.8–F22, and a resolution of 5 MP.

Key Features#

- Fits all Basler cameras with a sensor size of up to 1/2"

- Metal housing

- C-mount

- Low ghosting and veiling glare

- High contrast

- Locking screws for iris and focus

- Aperture indicators on iris ring and lens barrel

- Accurate focusing mechanism

Info

The technical data shown in the following sections are nominal design values. The real values of the delivered products can deviate from the nominal design values.

General Specifications#

|  | C125-0618-5M-P |
| --- | --- |
| Order Number | 2000034831 |
| Focal Length f' | 6.05 mm ± 5 % |
| Aperture Range | F1.8–F22 |
| Image Circle | 7.3 mm (1/2.5" format) |
| Focus Range | 0.1 m to infinity |
| Optimum Working Distance | 0.5 m |
| Optimum Magnification | 0.012 |
| Relative Illumination at Full Aperture | At least 55 % (see Relative Illumination versus Image Height) |
| Resolution (25 % MTF, Center, Full Aperture) | Designed for 230 LP/mm (2.2 µm pixel size, see Resolution versus Image Height) |
| Resolution at 5.8 mm Image Diagonal (80 % of a Basler acA2500-14gm/c camera or a Basler acA2500-14um/c camera) | At least 50 % of resolution in image center |
| Optical Distortion | Typical -9.9 % (barrel distortion, see Distortion versus Image Height) |
| Angle of View, 1/2.5" Format (using a Basler acA2500-14gm/c camera or a Basler acA2500-14um/c camera) | Horizontal: 52.4° (@ MOD) to 53.1° Vertical: 39.6° (@ MOD) to 40.1° |
| Angle of View, 1/3" Format (using a Basler acA1300-30gm/c camera or a Basler acA1300-30um/c camera) | Horizontal: 44.9° (@ MOD) to 45.5° Vertical: 33.6° (@ MOD) to 34.1° |
| Wavelength Range | Visible (400–700 nm, see Transmittance) |
| Pupil Magnification, ß'P | 3.95 |
| Chief Ray Angle, CRA | 8.2° |
| Front Focal Length, sF | 12.62 mm |
| Back Focal Length, s'F | 9.75 mm |
| Principal Point Separation, HH' | 22.45 mm |
| Entrance Pupil Position, sEP | 14.15 mm |
| Exit Pupil Position, s'AP | -14.16 mm |
| Overall Optical Length, d | 37.42 mm |